### Photomobile materials, innovative approaches: from soft robotics to solar energy harvesting through 4D process printing

Domenico Sagnellia, Fausta Loffredob, Anna De Girolamo Del Maurob, Amalia D’Avinoa, Bryan Guilcapia, Fulvia Villanib*, Tommaso Fasolinob, Maria Montaninob, Giuliano Sicob, Riccardo Misciosciab, Lucia Pettia, Giuseppe Nennab*

a Institute of Applied Sciences and Intelligent Systems, National Research Council, Via Campi Flegrei, 34, Pozzuoli, 80078, Naples, Italy

b ENEA Centro Ricerche Portici, Piazzale E. Fermi, 1, Portici, 80055, Naples, Italy

*Email address; giuseppe.nenna@enea.it Phone Number: +390817723410

ABSTRACT:

Among smart materials, photomobile materials (PMP) have emerged as a novel class that has attracted considerable attention due to their capability to respond to light stimuli and produce precise movements. A central feature of these materials is the incorporation of azobenzene moieties, which serve as primary driver of motion due to their ability to undergo reversible trans–cis photoisomerization when irradiated at specific wavelengths.

To improve the performances of PMP films, we investigated their photomechanical responsiveness, particularly the bending angles and actuation speed, under various conditions. We tried different strategies to improve thermal distribution by laminating the PMP film with a copper layer, enabling the material to withstand higher levels radiation without performance degradation. We incorporated different types of nanoparticles (e.g. carbon black, zinc oxides, silver nanocuboids, etc.) into the PMP polymer matrix, indirectly activating the azobenzene, to enlarge the spectral bandwidth usable for actuation and to increase actuation efficiency and reduce response time.

The dynamic behavior of PMP actuator was modelled over time and the results revealed that its response can be approximated by harmonic oscillator models, offering a predictive framework for the future design and application of these light-driven systems.

We investigated the potential of solar radiation to induce mechanical motion, and we extended the investigation to examine the self-oscillation properties of the PMPs under continuous light radiation. and their effects when coupled with piezoelectric devices (PZL), enabling the generation of electrical energy. The resulting electro-optical behaviours were discussed to provide guidelines for designing innovative opto-mechanical energy-harvesting system prototypes.

Finally, we present a novel materials-based strategy that bypasses alignment procedures by integrating ZnO nanoparticles into an LCE ink, enabling a simplified, direct-write 4D printing process. This approach offers a scalable pathway towards the rapid fabrication of complex, stimuli-responsive architectures for applications in soft robotics and adaptive systems.

BIOGRAPHY: 
Giuseppe Nenna received his Ph.D. in ‘New Technologies for Materials, Sensors and Imaging’ from Naples University “Federico II” in 2009. He is working as a senior researcher at the Italian National Agency for New Technologies, Energy and Sustainable Economic Development (ENEA). His activities are focused on Optics and Electronics, to study radiation-matter interaction, develop new optics elements and optoelectronic devices. He was involved in several Italian and European projects related to photonics, optoelectronic, sensors and photoresponsive materials. He participated in the creation of several patents based on optical instruments or optoelectronic systems.

Theme:

Type of presentation: PowerPoint Presentation

Mode of presentation: (In-Person)

All the abstracts should be submitted through online abstract submission (or) can be mailed to jamesmichael@materials-science.org